| ID: 01 |  | - Restrict access - |
| --- | --- | --- |
| Temporary relocation from residential areas |  |  |
| Objective | To reduce external gamma and beta doses from material deposited on surfaces and inhalation doses from material resuspended from surfaces within contaminated inhabited areas. |  |
| Other benefits | Any clean-up required will be easier to implement while the population is absent from the area. |  |
| Countermeasure description | The removal of people from a contaminated area to an area where external dose rates are significantly lower on a temporary basis. It is likely that people would be moved to an area that is sufficiently far outside the contaminated area that doses are minimal but is near enough for people to commute to their normal places of work. Temporary relocation should be time bound. Temporary relocation may also be considered while clean-up of a residential area is undertaken. |  |
| Target surface or population | People living in contaminated areas. |  |
| Target radionuclides | All radionuclides. Particularly useful for short-lived radionuclides. See Appendix B for information on radionuclides. |  |
| Scale of application | Any number of poeople, although much easier to implement on a small scale. |  |
| Timing of implementation | Maximum benefit if people are moved out soon after deposition or are evacuated during the emergency phase and do not return (see data sheet 6 for more information on evacuation). |  |
| Constraints on implementation |  |  |
| Legal | Compensation for people moved and possible lack of earnings. Provision of security for empty buildings. |  |
| Environmental / technical | Maintenance of buildings and environment for longer term temporary relocation. |  |
| Effectiveness |  |  |
| Reduction in contamination on the surface | This option will not reduce contamination levels in the restricted area |  |
| Reduction in surface dose rates |  |  |
| Reduction in resuspension |  |  |
| Averted doses | Doses will be averted during the period of relocation if people are moved fully away from the affected area. The averted dose will be influenced by the level of exposure at the new location. On returning to the location, people’s subsequent dose will, in part, depend on the nature of any clean-up measures that may have been undertaken whilst they were away. |  |
| Additional doses | People involved in implementing temporary relocation could be exposed to: - external exposure from deposited radioactive material - inhalation of radioactive material resuspended from the ground and other surfaces (may be enhanced over normal levels) No illustrative doses are provided as they will depend on e.g. the type of contamination, environmental conditions, the tasks undertaken by individuals, controls on working and the use of Personal Protective Equipment (PPE). |  |
| Factors influencing effectiveness of procedure (technical) |  |  |
| Factors influencing effectiveness of procedure (social) | Compliance with relocation, as people cannot be forced to leave their homes. Ability to prevent unauthorised access subsequently. |  |
| Requirements |  |  |
| Required specific equipment | Transport for moving people and possessions. |  |
| Required utilities and infrastructure | Alternative accommodation / housing. Infrastructure to support relocated populations: schools, doctors, social services etc. Security services for area that has been relocated. |  |
| Required consumables | Fuel and parts for vehicles and other transport |  |
| Required skills | Drivers. Security personnel may be required to support drivers. |  |
| Required safety precautions |  |  |
| Waste |  |  |
| Amount and type | None |  |
| Intervention costs |  |  |
| Equipment | Hiring vehicles for transporting people and possessions. Rental costs for accommodation/housing |  |
| Consumables | Fuel and parts for transport vehicles |  |
| Operator time | Assuming people are moved about 1 hour away, it is estimated that one person can relocate 60 people every 4 hours. Labour costs for bus / coach drivers and maintenance and security of relocated area. |  |
| Factors influencing costs | The following factors influence the time taken to implement the option and hence labour costs: - weather - type of vehicle used - number of vehicles available? - ease of access and transport route - distance people have to be moved - numbers of people being moved Availability of appropriate accommodation |  |
| Side effects / impact |  |  |
| Environmental impact | Increasing the size of the population in the area where people are temporarily relocated may impact on the environment, e.g. amount of general waste generated, increased traffic. |  |
| Social impact | Disruption in the affected communities (those moved and those in the receiving communities). Fragmentation of communities. Additional burden on schools, medical and recreational services |  |
| Practical experience | Some experience of temporary relocation for other incidents at a local level. Relocation after the Chernobyl accident. |  |
| Key references | Allen (1993) |  |