| ID: 14 |  | - Buildings - external surfaces - |
| --- | --- | --- |
| Roof brushing |  |  |
| Objective | To reduce external gamma and beta doses from contamination on roofs of buildings within inhabited areas, and reduce inhalation dose from material resuspended from roofs. |  |
| Other benefits | Will remove contamination from roofs of buildings. |  |
| Countermeasure description | The roof is cleaned using commercially available rotating brushes driven by compressed air at 700 litres/minute (water is applied at ordinary mains pressure). Cleaning is carried out in a closed (shielded) ‘box’ system. The device is mounted on an extendable rod that allows operation from the top of the roof or, in the case of single-storey buildings, from the ground. Contaminated waste can be segregated. Water can be filtered and recycled. Waste arising is largely solids, e.g. moss that is collected. Dust creation during implementation is unlikely to be a problem and and so methods are not required to reduce the resuspension hazard to workers. |  |
| Target surface or population | Roofs of buildings. |  |
| Target radionuclides | All radionuclides. Suitable for reducing doses from short-lived radionuclides if implemented quickly. See Appendix B for information on radionuclides. |  |
| Scale of application | Suitable for roofs of buildings. Can be done on a large scale if equipment is available. |  |
| Timing of implementation | Maximum benefit if carried out soon after deposition. However, brushing of roofs of buildings can be effective up to 10 years after deposition depending on roof material and level of removable debris on the roof. |  |
| ### Constraints on implementation |  |  |
| Legal | - Use on listed and other important buildings - Ownership and access to property - Solid waste disposal legislation |  |
| Environmental / technical | Severe cold weather (may require heating of water). |  |
| ### Effectiveness |  |  |
| Reduction in contamination on the surface | A decontamination factor (DF) of between 2 and 7 can be achieved. Repeated application is unlikely to provide any significant increase in DF. In the short term, the quoted DF range can be considered to be same for all radionuclides, with the exception of elemental iodine and tritium for which thorough washing of impermeable surfaces will lead to virtually full removal. |  |
| Reduction in surface dose rates | External gamma and beta dose rate contributions from roofs of buildings will be reduced by approximately the value of the DF. |  |
| Reduction in resuspension | Resuspended air concentrations above the roof surface can also be assumed to be reduced by the value of the DF. |  |
| Averted doses | Reductions in external gamma dose rate shortly after decontamination of the roof surfaces received by a member of the public living in and inhabited area could be expected to be in the range of 5-10 %. This is an illustrative value and should only be used to provide an indication of the likely effectiveness of this option and to compare across options. Further details can be found in Appendix C. Factors influencing dose reduction: - Consistency in effective implementation of option over entire area - Consistent application of option over the contaminated area (ie operator skill) - Careful implementation. Special care must be taken to clean roof gutters and drain pipes. Care should be taken to wash contamination to the roof gutter and not just move it around the roof). - Time of implementation. The impact of cleaning the surfaces on the overall doses will be reduced with time as there will be less contamination on the surfaces due to natural weathering. The longer the time between deposition and implementation of the option the less effective it will be - Population behaviour in area and the time spent by individuals close to or in buildings. - Amount of buildings in the area, i.e. environment type/land use. |  |
| Additional doses | Exposure pathways workers could be exposed to are: - External exposure from environment and contaminated equipment - Inhalation of radioactive material resuspended from the ground and other surfaces (may be enhanced over normal levels) - Inadvertent ingestion of dust from workers' hands Contributions from pathways in italics will not be significant and using personal protective equipment (PPE) can control doses from these pathways. Exposure routes from transport and disposal of waste are not included. Beta/gamma hazard: For radionuclides that present a beta/gamma hazard, external dose to workers from contamination in the environment will be a few times higher than public doses over the period of implementation. Even under very dusty conditions, the inhalation dose from resuspended material will only make a small contribution to the total worker dose. Alpha hazard: For radionuclides that present an alpha hazard, inhalation dose to workers from resuspended material will typically be a few times higher than public doses over the period of implementation. External dose from contamination in the environment can be ignored. For further information on worker doses, see Appendix D. |  |
| Factors influencing effectiveness of procedure (technical) | - Roof material - Amount of removable debris on roof, e.g. moss, pine needles - Water temperature |  |
| Factors influencing effectiveness of procedure (social) |  |  |
| ### Requirements |  |  |
| Required specific equipment | - Pressure washer with rotating brush attachment, filters and collection tank ( ~ €6000 + €1000 – 2000 for mobile compressor, if required). - Scaffolding and roof-ladders or fire-tender with hydraulic platform - Transportation vehicles for equipment and waste |  |
| Required utilities and infrastructure | - Roads for transport of equipment and waste - Water supply |  |
| Required consumables | - Water (15 l m-2) - Fuel for generators ( 5 l h-1 for mobile compressor) and transport vehicles. |  |
| Required skills | Skilled personnel essential for working at heights. |  |
| Required safety precautions | - Lifeline - Safety helmets - Water-proof clothing should be recommended. Precautions are needed to ensure that people making connections to mains water supplies do not inadvertently contaminate the water supply, e.g. by back-flow from vessels containing radioactivity or other contaminants, or operate hydrants in a way that disturbs settled deposits within the water main system. |  |
| ### Waste |  |  |
| Amount and type | Amount: 2 10-1 – 6 10-1 kg m-2 solid and 15 l m-2 water Type: Dust and moss (sludge) Amount of waste depends on amount of moss and other debris on the roof. Care must be taken not to block drains with moss etc. Water can be filtered and recycled It should be noted that waste may be toxic, for example containing asbestos. |  |
| Intervention costs (see Appendix E) |  |  |
| Equipment | € 6 m-2 |  |
| Consumables | No significant cost identified |  |
| Labour | € 9 m-2 |  |
| Operator time | 8 m2/team.hr (team size: 1 - 2 people) Work rate excludes setting up scaffolding |  |
| Factors influencing costs | The following factors influence the time taken to implement the option and hence labour costs: - Weather - Building height and pitch of roof - Type of equipment used - Access - Proximity of water supplies - Type of surface, numbers of gutters etc |  |
| ### Side effects / impact |  |  |
| Environmental impact | The disposal or storage of waste arising from the implementation of this option may have an environmental impact. However, this should be minimised through the control of any disposal route and relevant authorisations. |  |
| Social impact | - Brushing of roofs will make an area look clean; implementation may give public reassurance. |  |
| Practical experience | Tested on realistic scale on selected roofs of different types in the former Soviet Union after the Chernobyl accident. |  |
| Key references | Andersson (1996); Andersson et al (2003); Brown and Jones (2000); Brown, Charnock and Morrey (2003); Hubert et al (1996); NRPB (2005); Roed, Andersson and Prip (1995); Roed and Andersson (1996); Roed et al (1996) |  |