| ID: 22 |  |  | - Buildings - indoor surfaces - |  |
| --- | --- | --- | --- | --- |
| Vacuum cleaning |  |  |  |  |
| Objective |  | To reduce external gamma and beta doses arising from contamination on internal surfaces of buildings and indoor objects within inhabited areas, and reduce inhalation dose from material resuspended from these surfaces and objects. |  |  |
| Other benefits |  | Will remove contamination from indoor surfaces and objects in buildings. |  |  |
| Countermeasure description |  | Any domestic or industrial vacuum cleaner can be used to clean surfaces and objects, such as furniture. However, it is preferable to use a vacuum cleaner fitted with HEPA filters of 99% efficiency to 0.6 micron particles to prevent resuspension. Machines are electrically operated from mains electricity. This option is likely to give rise to dust, particularly in dusty environments. Using water to dampen the surface or the use of a tie-down material is unlikely to be practicable and so it will be necessary to provide personal protective equipment (PPE) for the workers to reduce the resuspension hazard. Other cleaning methods for indoor surfaces are discussed in data sheets 23 (washing) and 24 (other cleaning methods such as scrubbing and steam cleaning). |  |  |
| Target surface or population |  | Internal surfaces (particularly floors) and objects in buildings. |  |  |
| Target radionuclides |  | All radionuclides. Particularly suitable for removing short-lived radionuclides if implemented quickly. See Appendix B for information on radionuclides. |  |  |
| Scale of application |  | Suitable for indoor surfaces in all types of building.. |  |  |
| Timing of implementation |  | Maximum benefit if carried out within a few weeks of deposition when maximum contamination on surfaces. However, over longer periods, contamination may be brought into buildings eg on the soles of shoes, and so repeated application may be beneficial. |  |  |
| ### Constraints on impolementation |  |  |  |  |
| Legal |  | - Liabilities for possible damage to property - Ownership and access to property - Use in listed or other historic buildings and on precious objects. |  |  |
| Environmental / technical |  |  |  |  |
| ### Effectiveness |  |  |  |  |
| Reduction in contamination on the surface |  | Vacuum cleaning of carpets will generally have an insignificant effect on concentrations of contaminated particles in the region of size 1μm (as observed with the initial caesium contamination after the Chernobyl accident). However, a fraction of the contamination will rapidly become attached to larger house dust particles (>5 μm), for which vacuum cleaning can give effective removal. Soil particles brought into the buildings on shoes or by the wind will be relatively large and therefore easy to remove. A decontamination factor (DF) of between 5 and 10 can be achieved, although there is likely to be large variation in this value. The quoted range assumes that this option is implemented within a few weeks of deposition and no previous cleaning has taken place Repeated application is unlikely to provide any significant increase in DF. |  |  |
| Reduction in surface dose rates |  | External gamma and beta dose rates immediately above the cleaned surface will be reduced by a value similar to the DF. |  |  |
| Reduction in resuspension |  | Resuspended air concentrations will be reduced by a value similar to the DF. |  |  |
| Averted doses |  | Reductions in external gamma dose rate shortly after decontamination of the indoor building surfaces received by a member of the public living in an inhabited area could be expected to be around 15% following deposition under dry conditions. Following wet deposition, reductions in dose-rates will be negligible. This is an illustrative value and should only be used to provide an indication of the likely effectiveness of this option and to compare across options. Further details can be found in Appendix C. Factors influencing dose reductions: - Amount of time spent by individuals inside the buildings. - Consistency in effective implementation of option over entire area. - Weather at time of deposition; less material is deposited indoors during wet deposition. Deposition also influenced by the amount of furniture and furnishings in the buildings and ventilation rates. - Application of appropriate clean-up to other indoor surfaces and objects. - Time of implementation. The impact of cleaning the surfaces on the overall doses will be reduced with time as there will be less contamination on the surfaces due to natural weathering. |  |  |
| Additional doses |  | Exposure pathways workers could be exposed to are: - External exposure from environment and contaminated equipment - Inhalation of radioactive material resuspended from the floor and other surfaces (may be enhanced over normal levels) - Inadvertent ingestion of dust from workers' hands Contributions from pathways in italics will not be significant and using personal protective equipment (PPE) can control doses from these pathways. Exposure routes from transport and disposal of waste are not included. Beta/gamma hazard: For radionuclides that present a beta/gamma hazard, external dose to workers from indoor contamination will be approximately half that of public doses over the period of implementation. Even under very dusty conditions, the inhalation dose from resuspended material will only make a small contribution to the total worker dose. Alpha hazard: For radionuclides that present an alpha hazard, inhalation dose to workers from resuspended material will typically be more than 10 to 100 times lower than public doses over the period of implementation. If dust levels are enhanced, and workers are not protected, then over long periods of clean-up (i.e. over more than about six months), doses to workers may begin to approach those of the public. External dose from indoor contamination can be ignored. For further information on worker doses, see Appendix D. |  |  |
| Factors influencing effectiveness of procedure (technical) |  | - Type and condition of surface - Time of operation (the longer the time between deposition and implementation of the option the less effective it will be as contaminated dust may have migrated elsewhere) - Consistent application over the contaminated area; need to ensure edges and corners are cleaned - Amount of dust on surfaces at the time of deposition - Whether any cleaning has already been undertaken - Efficiency of equipment (depends on aerosol size of contaminant) - Ability to vacuum objects |  |  |
| Factors influencing effectiveness of procedure (social) |  |  |  |  |
| ### Requirements |  |  |  |  |
| Required specific equipment |  | - Vacuum cleaner with brush attachment and upholstery cleaning attachment (preferably HEPA filtered industrial vacuum cleaner). - Transport vehicles for equipment and waste. |  |  |
| Required utilities and infrastructure |  | - Mains electricity supply. - Roads for transport of equipment and waste. |  |  |
| Required consumables |  | - Fuel and parts for transport vehicles. |  |  |
| Required skills |  | Only a little instruction is likely to be required. |  |  |
| Required safety precautions |  | Personal protective equipment (PPE), including respiratory protection, will be required because dust may be produced. |  |  |
| ### Waste |  |  |  |  |
| Amount and type |  | Amount: 5 10-3 kg m-2 Type: Dust Contaminated filters (40 g m-2 per year) which may have high contamination levels. |  |  |
| Intervention costs (see Appendix E) |  |  |  |  |
|  |  | Hard smooth surfaces (e.g. tiles, linoleum, wood) |  | Carpets, rugs |
| Equipment € m-2 |  | 2 10-3 |  | 6 10-3 |
| Consumables € m-2 |  | No significant cost identified |  |  |
| Labour € m-2 |  | 3 10-1 |  | 6 10-1 |
|  |  | Note that costs for cleaning upholstery and soft furnishings are likely to be about a factor of 2 higher |  |  |
| Operator time | Work rate m2/team.hr | 1.2 102 – 1.5 102 Work rate for cleaning upholstery and soft furnishings is about 25m2 h-1 |  |  |
|  | Team size people | 1 |  |  |
| Factors influencing costs |  | The following factors influence the time taken to implement the option and hence labour costs: - Building size - Type of equipment used - Access - Use of personal protective equipment (PPE) - Tidiness of houses and amount of ‘contents’ - Amount of dust/dirt on surfaces |  |  |
| ### Side effects / impact |  |  |  |  |
| Environmental impact |  | The disposal or storage of waste arising from the implementation of this option may have an environmental impact. However, this should be minimised through the control of any disposal route and relevant authorisations. |  |  |
| Social impact |  | Possible damage to building surfaces and objects Positive benefit of cleaning houses. |  |  |
| Practical experience |  | Several small scale tests have been reported before/after the Chernobyl accident in 1986. |  |  |
| Key references |  | Allott, Kelly and Hewitt (1994); Andersson et al (2003); Brown and Jones (2000); Brown, Charnock and Morrey (2003); NRPB (2005); Roed (1985); Tschiersch (1995) |  |  |