| ID: 25 |  | - Buildings - indoor surfaces - |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Surface removal |  |  |  |  |  |
| Objective | To reduce external gamma and beta doses arising from contamination on indoor surfaces of buildings (primarily floors, walls and ceilings) within inhabited areas, and to reduce inhalation dose from material resuspended from those surfaces. |  |  |  |  |
| Other benefits | Will remove contamination from indoor surfaces in buildings. |  |  |  |  |
| Countermeasure description | Paint can be removed from underlying plaster using commercial sanders. The technique is likely to produce a lot of dust. However, it may be possible to control this using an improvised vacuum shroud placed around the sander which is connected to a vacuum cleaner (preferably with HEPA filter). Plaster can be removed using long-reach pneumatic chisels. Wallpaper glued directly to plaster can be removed by manual scraping or using steam strippers. Wallpaper glued directly to plaster can be removed by manual scraping or by using steam strippers. Wallpaper with a paper backing (integral to the paper or separate) can be peeled easily from the backing paper by hand. Backing paper will need to be removed by manual scraper or steam stripper, if necessary. Linoleum and carpet not stuck to floors and are relatively easily removed manually. Linoleum tiles can be stuck to concrete floors or stuck to hardboard which is pinned to a wooden floor. Machinery may be required to remove tiles stuck to concrete floors. For tiles stuck to hardboard, removal involves removing both the hardboard and tiles together by removing the pins and pulling the hardboard away from the floor. Wooden floors are removed by prising the floor boards from the cross joints which are then themselves removed using saws. |  |  |  |  |
| Target surface or population | Indoor surfaces (primarily floors, walls or ceilings) of buildings. |  |  |  |  |
| Target radionuclides | All radionuclides. Unlikely to be considered for removal of short-lived radionuclides alone. See Appendix B for information on radionuclides. |  |  |  |  |
| Scale of application | Suitable for small areas of indoor surfaces in all types of building.. |  |  |  |  |
| Timing of implementation | Maximum benefit if carried out within a few weeks of deposition when maximum contamination on surfaces. |  |  |  |  |
| ### Constraints on implementation |  |  |  |  |  |
| Legal | - Liabilities for possible damage to property - Ownership and access to property - Use in listed or other historic buildings and on precious objects. |  |  |  |  |
| Environmental / technical |  |  |  |  |  |
| ### Effectiveness |  |  |  |  |  |
| Reduction in contamination on the surface | If carried out carefully, these removal processes can remove virtually all the contamination on the surface. However, the process of removing paper, paint or plaster may result in the spread of contamination onto other surfaces via dust. Repeated application is unlikely to provide any significant increase in DF if implemented thoroughly the first time. |  |  |  |  |
| Reduction in surface dose rates | No estimates made. |  |  |  |  |
| Reduction in resuspension | No estimates made. |  |  |  |  |
| Averted doses | Dose reductions have not been estimated for this option. Some indication of possible dose reductions can be found in data sheet 23. However, it should be noted that removal of surfaces will only reduce doses to people while they are indoors and will be very dependent on the specific situation and the surfaces cleaned. Factors influencing dose reduction: - Consistency in effective implementation of option over entire area. - Weather at time of deposition; less material is deposited indoors during wet deposition. Also, deposition is influenced by the amount of furniture and furnishings and ventilation rates. - Application of appropriate clean-up to other indoor surfaces and objects. - Time of implementation. The impact of cleaning the surfaces on the overall doses will be reduced with time as there will be less contamination on the surfaces due to natural weathering and cleaning. - Care of application. Need to remove contamination from surfaces and not just move it around the surface or onto another surface. - Amount of time people spend in the buildings. |  |  |  |  |
| Additional doses | Exposure pathways workers could be exposed to are: - External exposure from environment and contaminated equipment - Inhalation of radioactive material resuspended from the floor and other surfaces (may be enhanced over normal levels) - Inadvertent ingestion of dust from workers' hands Contributions from pathways in italics will not be significant and using personal protective equipment (PPE) can control doses from these pathways. Exposure routes from transport and disposal of waste are not included. Beta/gamma hazard: For radionuclides that present a beta/gamma hazard, external dose to workers from indoor contamination will be approximately half that of public doses over the period of implementation. Even under very dusty conditions, the inhalation dose from resuspended material will only make a small contribution to the total worker dose. Alpha hazard: For radionuclides that present an alpha hazard, inhalation dose to workers from resuspended material will typically be more than 10 to 100 times lower than public doses over the period of implementation. If dust levels are enhanced, and workers are not protected, then over long periods of clean-up (i.e. over more than about six months), doses to workers may begin to approach those of the public. External dose from indoor contamination can be ignored. For further information on worker doses, see Appendix D. |  |  |  |  |
| Factors influencing effectiveness of procedure (technical) | - Type and condition of surface - Time of operation (the longer the time between deposition and implementation of the option the less effective it will be as contaminated dust may have migrated elsewhere) - Consistent application over the contaminated area; need to ensure all the surface material is removed - Amount of dust on surfaces at the time of deposition - Whether any cleaning has already been undertaken - Collection of all removed surface material |  |  |  |  |
| Factors influencing effectiveness of procedure (social) |  |  |  |  |  |
| ### Requirements |  |  |  |  |  |
| Required specific equipment | - Scrapers - Steam strippers - Pneumatic chisels - Removing lino tiles from concrete: machine (long reach scaler) to remove tiles stuck to concrete floors - Saws for removing wooden floors - Transport vehicles for equipment and waste |  |  |  |  |
| Required utilities and infrastructure | - Mains electricity supply. - Water supply - Roads for transport of equipment and waste. |  |  |  |  |
| Required consumables | - Fuel and parts for transport vehicles. - Water and detergent |  |  |  |  |
| Required skills | Only a little instruction is likely to be required. |  |  |  |  |
| Required safety precautions | Gloves and overalls. Waterproof clothing may be required. Personal protective equipment (PPE) may be required under dusty conditions to reduce the hazard from resuspension. Appropriate safety measures and respiratory protection will be required if asbestos is present. |  |  |  |  |
| ### Waste |  |  |  |  |  |
| Amount and type | Surface removed |  |  | Amount, kg m-2 solid waste | Type |
|  | Wallpaper |  |  | 1.0 | Wallpaper |
|  | Paint |  |  | 1.0 | Paint and plaster dust |
|  | Plaster |  |  | 1 101 | Plaster |
|  | Carpet |  |  | 4 10-1 | Carpet |
|  | Linoleum / linoleum tiles (laid on concrete) |  |  | 3.0 | Linoleum, tiles, cement |
|  | Linoleum tiles (laid on wood) |  |  | 4 | Tiles and hardboard |
|  | Wood floor |  |  | 7 | Wood |
|  | Any water resulting from steam stripping will not be able to be collected and so floor surfaces will need to be covered and covering disposed of. |  |  |  |  |
| Intervention costs (see Appendix E) |  |  |  |  |  |
| Equipment costs | Surface removed |  |  | Equipment cost, € m-2 |  |
|  | Wallpaper |  |  | 8 10-4 (scraping) 2 10-4 (scraping and peeling) No significant cost for peeling alone |  |
|  | Paint |  |  | 9 10-2 (walls) 1 10-1 (ceilings) |  |
|  | Plaster |  |  | 1 (walls and ceilings) |  |
|  | Carpet |  |  | 8 10-2 |  |
|  | Linoleum |  |  | No significant costs |  |
|  | Linoleum tiles (laid on concrete) |  |  | 2 10-1 |  |
|  | Linoleum tiles (laid on wood) |  |  | No significant costs |  |
|  | Wood floor |  |  | No significant costs |  |
| Material costs | No significant costs identified. |  |  |  |  |
| Labour costs | Surface removed |  |  | Labour cost, € m-2 |  |
|  | Wallpaper |  |  | 6 10-1 (scraping) 2 10-1 (scraping and peeling) 9 10-2 (peeling) |  |
|  | Paint |  |  | 7 (walls) 9 (ceilings) |  |
|  | Plaster |  |  | 1 (walls and ceilings) |  |
|  | Carpet |  |  | 7 10-1 |  |
|  | Linoleum |  |  | 5 10-1 |  |
|  | Linoleum tiles (laid on concrete) |  |  | 2 |  |
|  | Linoleum tiles (laid on wood) |  |  | 2 10-1 |  |
|  | Wood floor |  |  | 1 |  |
| Operator time | Surface removed |  | Work rate, m2/team.hr |  |  |
|  | Wallpaper |  | 60 (scraping) 230 (scraping and peeling) 400 (peeling) |  |  |
|  | Paint |  | 5 (walls) 4 (ceilings) |  |  |
|  | Plaster |  | 25 (walls and ceilings) |  |  |
|  | Carpet |  | 100 |  |  |
|  | Linoleum |  | 80 |  |  |
|  | Linoleum tiles (laid on concrete) |  | 20 |  |  |
|  | Linoleum tiles (laid on wood) |  | 200 |  |  |
|  | Wood floor |  | 3 |  |  |
|  | Team size: people 2 for carpet removal; 1 for all other techniques |  |  |  |  |
| Factors influencing costs | The following factors influence the time taken to implement the option and hence labour costs: - Building size - Type of equipment used - Access - Use of personal protective equipment (PPE) - Tidiness of houses and amount of ‘contents’ - Thickness of surface covering/layers of wallpaper and/or paint |  |  |  |  |
| ### Side effects / impact |  |  |  |  |  |
| Environmental impact | The disposal or storage of waste arising from the implementation of this option may have an environmental impact. However, this should be minimised through the control of any disposal route and relevant authorisations. |  |  |  |  |
| Social impact | Possible damage to building surfaces Positive benefit of cleaning houses. |  |  |  |  |
| Practical experience | There is no readily available evidence of any practical experience of the use of this option for clean-up of radioactive contamination in inhabited areas. |  |  |  |  |
| Key references | Brown and Jones (2000); Brown, Charnock and Morrey (2003); NRPB (2005) |  |  |  |  |