| ID: 28 |  | - Buildings - precious objects - |
| --- | --- | --- |
| Storage, shielding, covering, gentle cleaning |  |  |
| Objective | To reduce external gamma and beta doses arising from contamination on personal and precious objects within inhabited areas, and reduce inhalation doses from material resuspended from these objects. This option is likely to be implemented primarily for public reassurance as exposure from personal and precious objects is unlikely to be a significant contribution to an individuals dose. |  |
| Other benefits | Gentle cleaning will remove contamination from precious objects within buildings. |  |
| Countermeasure description | It may not be possible or appropriate to carry out decontamination of precious objects, such as museum artefacts, tapestries, jewellery, paintings etc because of the risk of damaging the objects during the cleaning process. Several alternative options are available for such objects. - If objects are placed within rooms or storage facilities to which people do not have general access, then significant reductions in dose rates to people in adjoining rooms and buildings can be achieved. - Some objects, which do not require handling, could be shielded or covered. For instance, museum artefacts could be placed behind leaded glass or perspex; they can remain on display, but the public will be shielded from the contamination. - Specialist, gentle cleaning techniques could be carried out on objects. |  |
| Target surface or population | Precious and personal objects within buildings. |  |
| Target radionuclides | All radionuclides. The storage option will be particularly suitable for short-lived radionculides. Shielding and covering will be particularly effective for beta emitters. See Appendix B for information on radionuclides. |  |
| Scale of application | Particularly suitable for small objects. |  |
| Timing of implementation | Maximum benefit if carried out soon after deposition. |  |
| ### Constraints on implementation |  |  |
| Legal | - Liabilities for possible damage to objects - Ownership and access to objects - Use in listed or other historic buildings |  |
| Environmental / technical |  |  |
| ### Effectiveness |  |  |
| Reduction in contamination on the surface | Contamination on the surface of objects will only be reduced if gentle cleaning is applied. |  |
| Reduction in surface dose rates | Cleaning will reduce surface doses rates from objects. Shielding and storage will reduce external gamma and beta dose rates; the degree of reduction will depend on the thickness of shielding used. Some examples are given below: Brick or concrete wall thicknesses of 10-20 cm will half the dose rate outside a room for medium to high energy gamma emitters. Around 10 mm lead will be sufficient to half the gamma dose rate for many radionuclides and a few centimetres could reduce gamma dose-rates by a factor of 10. For steel, these thicknesses would need to be at least doubled to receive the same level of shielding. 1-5mm of glass will totally absorb beta particles for the range of beta energies likely to be of concern. Plastic (perspex) would need to be about twice as thick to have the same effect. Air can also be used as a shielding material. 1-2 m of air will reduce dose-rates to very low levels for weak beta emitters: a distance of up to 10 m would be needed to give high reductions in dose rate for high energy beta emitters such as 90Sr/90Y. For gamma emitters, dose rates will drop off in air in proportion to the square of the distance, eg, if people are kept 5m away from an object, the dose-rate they receive from that object will be 25 times lower than if they were 1m away. |  |
| Reduction in resuspension | Removing contamination from an object's surface will reduce the contamination available for resuspension. Shielding an object in a closely fitting container will stop all resuspension. |  |
| Averted doses | Dose reductions have not been estimated for this option. It should be noted that cleaning objects will only reduce doses to people while they are indoors and will be very dependent on the specific situation and the objects and other surfaces cleaned. Factors influencing dose reduction: - Consistency in effective implementation of option over contamianted area. - Weather at time of deposition; less material is deposited indoors during wet deposition. - Application of appropriate clean-up to other indoor surfaces and objects. - Time of implementation. The impact of cleaning the surfaces on the overall doses will be reduced with time as there will be less contamination on the surfaces due to natural weathering. - The amount of time spent by individuals inside the buildings. - Care of application. Need to wash contamination off surfaces and not just move it around the surface or onto another surface. |  |
| Additional doses | Exposure pathways workers could be exposed to are: - External exposure from environment and contaminated equipment - Inhalation of radioactive material resuspended from the floor and other surfaces (may be enhanced over normal levels) - Enhanced resuspension of activity deposited in the indoor environment leading to inhalation of dust generated - Inadvertent ingestion of dust from workers' hands Contributions from pathways in italics will not be significant and using personal protectiver equipment (PPE) can control doses from these pathways. Exposure routes from transport and disposal of waste are not included. Beta/gamma hazard: For radionuclides that present a beta/gamma hazard, external dose to workers from indoor contamination will be approximately half that of public doses over the period of implementation. Even under very dusty conditions, the inhalation dose from resuspended material will only make a small contribution to the total worker dose. Alpha hazard: For radionuclides that present an alpha hazard, inhalation dose to workers from resuspended material will typically be more than 10 to 100 times lower than public doses over the period of implementation. If dust levels are enhanced, and workers are not protected, then over long periods of clean-up (i.e. over more than about six months), doses to workers may begin to approach those of the public. External dose from indoor contamination can be ignored. For further information on worker doses, see Appendix D. |  |
| Factors influencing effectiveness of procedure (technical) | - Type, condition and fragility of object - Time of operation (for cleaning, the longer the time between deposition and implementation of the cleaning, the less effective it will be as contamination may have migrated elsewhere) - Consistent application of cleaning over entire object; need to ensure entire object's surface is cleaned. - Amount of dust on the surface of the object at the time of deposition. - Whether any cleaning has already been undertaken. - Weight of shielding material that can be used and any need to be able to view objects clearly. |  |
| Factors influencing effectiveness of procedure (social) |  |  |
| ### Requirements |  |  |
| Required specific equipment | Specialist cleaning equipment for gentle cleaning. Specialist lifting gear, if object is to be moved into storage. |  |
| Required utilities and infrastructure | Power and water supplies Storage facilities |  |
| Required consumables | Shielding materials, e.g. leaded glass if object is to be shielded. |  |
| Required skills | Specialist cleaning skills Specialist handling skills. |  |
| Required safety precautions | Gloves and overalls. |  |
| ### Waste |  |  |
| Amount and type | Waste water will be generated from cleaning. Quantities are unlikely to be large. |  |
| Intervention costs (see Appendix E) |  |  |
| Equipment | No significant costs identified. |  |
| Consumables | Costs of shielding materials and materials for reconstructing displays etc. |  |
| Operator time | Cleaning of precious objects is likely to take significantly longer than normal cleaning (see data sheet 23) |  |
| Factors influencing costs |  |  |
| ### Side effects / impact |  |  |
| Environmental impact |  |  |
| Social impact | Possible damage of objects with particular heritage significance. Lack of access to objects and buildings by the public |  |
| Practical experience | There is no readily available evidence of any practical experience of the use of this option for clean-up of radioactive contamination in inhabited areas. |  |
| Key references | Brown (2004); Delacroix et al (2002); Crick and Dimbylow (1985) |  |