| ID: 32 |  |  |  | - Roads and paved areas - |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Surface removal and replacement |  |  |  |  |  |  |  |
| Objective |  | To reduce external gamma and beta doses from contamination on roads, paved and other outdoor areas with ‘hard’ surfaces within inhabited areas, and reduce inhalation doses from material resuspended from these surfaces. |  |  |  |  |  |
| Other benefits |  | Will remove contamination from roads, paved and other outdoor areas with ‘hard’ surfaces within inhabited areas. |  |  |  |  |  |
| Countermeasure description |  | The most common forms of hard outdoor surfaces will be tarmac or concrete slabs. Standard machinery to remove asphalt surfaces is available in different sizes. They have a rotating drum with cutting teeth which conveys planed material (about 40 mm thick) to the middle of drum where it is pushed on to a conveyor belt and from there to flat bed truck. If machines do not have brushes for debris collection, this must be added or manual sweeping carried out. Water is sprayed continuously on to the drum to suppress dust. A typical highway maintenance machinery can remove a width of about 2 m per pass. Replacing/resurfacing asphalt and concrete roads can be undertaken using standard equipment. For replacement in small areas, manual methods are likely to be used, ie tarmac is deposited in several places and spread by shovel and rake, then tamped. For small surface areas it may also be possible to use a jackhammer to loosen existing tarmac and rubble can be shovelled into wheelbarrows. However, this has not been trialled. A small excavator/‘bob-cat’ can be used to remove concrete slabs. Concrete slabs are replaced by hand. The need to resurface asphalt and concrete surfaces will depend on the depth removed and other factors, such as acceptability. The area can be repaved with hot rolled asphalt or concrete paving machine to relay concrete. This option is likely to give rise to dust, so application of water to dampen the surface or the use of a tie-down material (see data sheet 34) is recommended prior to implementation to limit the resuspension hazard. |  |  |  |  |  |
| Target surface or population |  | Hard outdoor surfaces (roads, pavements, paths, yards, playgrounds etc.). |  |  |  |  |  |
| Target radionuclides |  | All long-lived radionuclides. Should not be considered for removal of short-lived radionuclides alone. See Appendix B for information on radionuclides. |  |  |  |  |  |
| Scale of application |  | Any size. Suitable for small surface areas (eg pavements, playgrounds) and large surface areas (eg roads) |  |  |  |  |  |
| Timing of implementation |  | Maximum benefit if carried out soon after deposition when maximum contamination is on the surfaces. However surface removal can be effective up to 10 years after deposition. |  |  |  |  |  |
| ### Constraints on implementation |  |  |  |  |  |  |  |
| Legal |  | - Liabilities for possible damage to property - Ownership and access to property - Use in conservation areas or at listed sites |  |  |  |  |  |
| Environmental / technical |  | If the surface of the road is cambered the removal depth will not be uniform. |  |  |  |  |  |
| ### Effectiveness |  |  |  |  |  |  |  |
| Reduction in contamination on the surface |  | A decontamination factor (DF) of between 5 and 10 can be achieved. Repeated application is unlikely to provide any significant increase in DF. |  |  |  |  |  |
| Reduction in surface dose rates |  | External gamma and beta dose rates and resuspension above the decontaminated surface will be reduced by the value of the DF. |  |  |  |  |  |
| Reduction in resuspension |  | Resuspended air concentrations above the surface will be reduced by the value of the DF. |  |  |  |  |  |
| Averted doses |  | Reductions in external gamma dose rate shortly after decontamination of the roof surfaces received by a member of the public living in and inhabited area could be expected to be in the range of 5-15 %. This is an illustrative value and should only be used to provide an indication of the likely effectiveness of this option and to compare across options. Further details can be found in Appendix C. Factors influencing dose reduction: - Consistency in effective implementation of option over a large area - Time spent by individuals on or close to ‘hard’ outdoor surfaces. - Amount of hard outdoor surfaces in the area ie environment type/land use - Time of implementation. The impact of cleaning the surfaces on the overall doses will be reduced with time as there will be less contamination on the surfaces due to natural weathering. - Whether decontamination is carried out on adjacent surfaces. |  |  |  |  |  |
| Additional doses |  | Exposure pathways workers could be exposed to are: - External exposure from environment and contaminated equipment - Inhalation of radioactive material resuspended from the ground and other surfaces (may be enhanced over normal levels) - Inadvertent ingestion of dust from workers' hands Contributions from pathways in italics will not be significant and using personal protective equipment (PPE) can control doses from these pathways. Exposure routes from transport and disposal of waste are not included. Beta/gamma hazard: For radionuclides that present a beta/gamma hazard, external dose to workers from contamination in the environment will be a few times higher than public doses over the period of implementation. Even under very dusty conditions, the inhalation dose from resuspended material will only make a small contribution to the total worker dose. Alpha hazard: For radionuclides that present an alpha hazard, inhalation dose to workers from resuspended material will typically be a few times higher than public doses over the period of implementation. External dose from contamination in the environment can be ignored. For further information on worker doses, see Appendix D. |  |  |  |  |  |
| Factors influencing effectiveness of procedure (technical) |  | - Evenness and condition of roads - Operator skill - Ineffective removal of contamination around drains and in gutters - Removal of loose debris from surface. |  |  |  |  |  |
| Factors influencing effectiveness of procedure (social) |  |  |  |  |  |  |  |
| ### Requirements |  |  |  |  |  |  |  |
| Required specific equipment |  | The equipment used for surface removal and replacement will depend on the size of the area being treated. |  |  |  |  |  |
|  |  | Small areas |  |  |  |  |  |
|  |  | - Small scale planer - Shovel |  |  | - Wheelbarrow - Tamper - Lorry |  |  |
|  |  | Large areas |  |  |  |  |  |
|  |  | - Planer with conveyor - Paving machine - Road sweeper |  |  | - Roller - JCB - Lorry |  |  |
|  |  | Transport vehicles for equipment and waste. |  |  |  |  |  |
| Required utilities and infrastructure |  | Roads for transport of equipment, materials and waste. |  |  |  |  |  |
| Required consumables |  | - Tarmac or concrete or concrete paving slabs - Tungsten carbide teeth - Fuel and parts for equipment, generators and vehicles |  |  |  |  |  |
| Required skills |  | Skilled personnel essential to operate equipment. |  |  |  |  |  |
| Required safety precautions |  | - Gloves - Safety goggles - Safety helmets - Respiratory protective equipment (RPE) |  |  |  |  |  |
| ### Waste |  |  |  |  |  |  |  |
| Amount |  | Asphalt: about 15 kg m-2 per cm removed Paving slabs (concrete): about 30 kg m-2 per cm removed Waste depends on thickness removed and density of material (paving slabs more dense than asphalt) |  |  |  |  |  |
| Type |  | Paving slabs, concrete and asphalt |  |  |  |  |  |
| Intervention costs (see Appendix E) |  |  |  |  |  |  |  |
|  |  | Surface removal only |  |  |  | Surface removal and replacement |  |
|  |  | Paving slabs | Asphalt |  |  | Paving slabs | Asphalt |
| Equipment € m-2 |  | 6 10-1 | 2 10-1 |  |  | 3 | 3 10-1 |
| Consumables € m-2 |  | 0 | 1 10-1 |  |  | 1 101 | 1 101 |
| Labour € m-2 |  | 3 | 2 10-1 |  |  | 2 101 | 1 |
| Operator time | Work rate m2/team.hr | Asphalt: 4 102 – 1 103 Paving slabs (concrete): 4 – 30 |  |  |  |  |  |
|  | Team size people | Asphalt: 2 – 4 Paving slabs (concrete): 2 Team of 14 needed if road surface replaced and a team of 4 for paving slab replacement |  |  |  |  |  |
| Factors influencing costs |  | The following factors influence the time taken to implement the option and hence labour costs: - Weather - Evenness and condition of surface (affects grinding depth) - Size of area to be treated - Type of equipment used / planer size / sweeping equipment - Access - Use of personal protective equipment (PPE) |  |  |  |  |  |
| ### Side effects / impact |  |  |  |  |  |  |  |
| Environmental impact |  | Road and pavement condition may be improved providing tarmac or concrete has been laid properly. The disposal or storage of waste arising from the implementation of this option may have an environmental impact. However, this should be minimised through the control of any disposal route and relevant authorisations. |  |  |  |  |  |
| Social impact |  | - Method of disposing such a large quantity of contaminated waste may not be acceptable to local residents. - Disruption of access if people remain in the area - May improve road conditions. |  |  |  |  |  |
| Practical experience |  | Tested on a small scale in the CIS, pre-Chernobyl tests in the USA. |  |  |  |  |  |
| Key references |  | Andersson (1996); Andersson and Roed (1999); Andersson et al (2003); Barbier and Chester (1990); Brown and Jones (2000); Brown, Charnock and Morrey (2003); Calvert et al (1984); NRPB (2005); Roed (1990); Roed, Andersson and Prip (1995) |  |  |  |  |  |