| ID: 39 |  | - Soil, grass and plants - |
| --- | --- | --- |
| Top soil and turf removal (manual) |  |  |
| Objective | To reduce external beta and gamma doses from contamination on outdoor grassed and soil areas within inhabited areas, and reduce inhalation doses from material resuspended from these areas. |  |
| Other benefits | Will remove contamination from grassed and soil areas. Removal of activity from grass areas in gardens may reduce subsequent contamination of soil used for growing food. This in turn may reduce up-take to food crops grown. |  |
| Countermeasure description | Turf and the top 50mm of topsoil are removed using (typically) a spade. Any plants and shrubs may need to be removed first. Optionally, the soil can be replaced and can be reseeded or returfed depending on the size of the area. This option is likely to give rise to dust, so application of water to dampen the surface or the use of a tie-down material is recommended prior to implementation to limit the resuspension hazard if removal is implemented in the first few months following deposition.(see data sheet 41). In the longer term, most of the contamination is attached to soil particles and is not in the respirable range. |  |
| Target surface or population | Grass surfaces in gardens, parks, playing fields and other small open spaces. Not recommended on land that has been tilled since the incident occurred. [Tilled areas can be treated but the waste volume will be much larger, as a greater depth of soil will have to be removed.] |  |
| Target radionuclides | All long-lived radionuclides. Should not be considered for removal of short-lived radionuclides alone. See Appendix B for information on radionuclides. |  |
| Scale of application | Suitable for small areas (e.g. residential gardens and small public gardens). |  |
| Timing of implementation | Maximum effectiveness will be achieved for several years after deposition has occurred since most contaminants migrate only very slowly down the soil profile (e.g. caesium remains in the top few centimetres for many years in the case of clay and brown earth soils). Measurements of contamination levels as a function of depth in the soil could be used to determine the depth of contamination and the likely effectiveness of 50 mm removal. Soil could be removed to a greater depth. May be beneficial to wait until after first rain so that most of dust has washed off other outdoor surfaces and buildings onto soil and grass areas. |  |
| ### Constraints on implementation |  |  |
| Legal | - Liabilities for possible damage to property - Ownership and access to property - Waste disposal of collected waste - Use on listed or historic sites and conservation areas |  |
| Environmental / technical | - Severe cold weather (i.e. frost or snow cover) - Soil texture, eg large rocks - In extreme cases, the slope of the area maybe a constraint |  |
| ### Effectiveness |  |  |
| Reduction in contamination on the surface | A decontamination factor (DF) of between 10 and 30 can be achieved if the removal depth is optimised.. If a standard removal depth is used, the effectiveness will reduce in time after this as contamination migrates to deeper soil depths. |  |
| Reduction in surface dose rates | External gamma and beta dose rates above the soil surface will be reduced by approximately the value of the DF. |  |
| Reduction in resuspension | Resuspended air concentrations above the surface will be reduced by the value of the DF. |  |
| Averted doses | Reductions in external gamma dose rate shortly after decontamination of the soil/grass surface received by a member of the public living in an inhabited area could be expected to be around 30% following deposition under dry conditions. Following wet deposition, reductions in dose rate could be expected to be higher, around 65%. These values are likely to be optimistic if there are also grass areas present that are not treated, eg in residential gardens. This is an illustrative value and should only be used to provide an indication of the likely effectiveness of this option and to compare across options. Further details can be found in Appendix C. Factors influencing dose reduction: - Consistency in effective implementation of option over the area. - Reductions in external doses and inhalation doses from resuspended material received by a member of public living in the area, will depend on the amount of the area these grassed areas. - Time of implementation. The impact of removing contaminated turf/soil surfaces on the overall doses will be reduced with time as there will be less contamination on the surfaces due to natural weathering. - Whether adjacent grass and other ground surfaces are also decontaminated. |  |
| Additional doses | Exposure pathways workers could be exposed to are: - External exposure from environment and contaminated equipment - Inhalation of radioactive material resuspended from the ground and other surfaces (may be enhanced over normal levels) - Inadvertent ingestion of dust from workers' hands Contributions from pathways in italics will not be significant and using personal protective equipment (PPE) can control doses from these pathways. Exposure routes from transport and disposal of waste are not included. Beta/gamma hazard: For radionuclides that present a beta/gamma hazard, external dose to workers from contamination in the environment will be a few times higher than public doses over the period of implementation. Even under very dusty conditions, the inhalation dose from resuspended material will only make a small contribution to the total worker dose. Alpha hazard: For radionuclides that present an alpha hazard, inhalation dose to workers from resuspended material will typically be a few times higher than public doses over the period of implementation. External dose from contamination in the environment can be ignored. For further information on worker doses, see Appendix D. |  |
| Factors influencing effectiveness of procedure (technical) | - Weather conditions, particularly those at the time of deposition, and the amount of rain after deposition. - Correct implementation of option – all soil, and any associated vegetation, must be collected to achieve the DF value quoted. Once contamination has migrated below 50mm in depth, the technique will start to become less effective unless the depth of removal is increased. This is likely to take several years after deposition. - Soil texture – dry, crumbly soils will be more difficult to remove completely - Evenness of ground - Uniformity of distribution of contamination with depth. |  |
| Factors influencing effectiveness of procedure (social) |  |  |
| ### Requirements |  |  |
| Required specific equipment | The equipment used will depend on the size of the area being treated. - Spade - Seeding machine (if required) - Transport vehicles for equipment and waste |  |
| Required utilities and infrastructure | Roads for transport of equipment, materials and waste. |  |
| Required consumables | - Fuel and parts for vehicles and equipment - Top soil - Plants and turf or grass seed (if required). |  |
| Required skills | Only a little instruction is likely to be required. Care must be taken to remove soil to the optimal depth. |  |
| Required safety precautions | Under very dusty conditions respiratory protection and protective clothes/gloves may be recommended to reduce the hazard from resuspended activity. |  |
| ### Waste |  |  |
| Amount and type | Amount: 5.5 101 – 7 101 kg m-2 if 5cm depth removed. Type: Soil and turf Segregation of contaminated waste is likely to be difficult. Surface contamination may be diluted within the removed 50 mm. Monitoring of waste to determine if it meets current waste disposal criteria will be important to ensure that the quantity of waste requiring special management is minimised. Waste depends on whether soil or soil and grass are collected. If the area has been tilled since contamination, a greater depth of soil will need to be removed and hence a greater amount of waste will be produced. |  |
| Intervention costs (see Appendix E) |  |  |
| Equipment € m-2 | 9 10-1 |  |
| Consumables € m-2 | 0 |  |
| Labour € m-2 | 2 |  |
|  | Total costs (€ m-2) including the additional work of: Replacing soil: 6 Replacing soil and returf: 1 101 Replacing soil and reseed: 6 |  |
| Operator time: | 1 101 m2/team.hr Team size: 1 to remove topsoil and turf. Up to 4 people if additional work is carried out. |  |
| Factors influencing costs | The following factors influence the time taken to implement the option and hence costs: - Soil type, condition and depth removed - Amount of vegetation to be removed - Weather - Topography - Size of area - Evenness of ground surface - Type of equipment used - Access |  |
| ### Side effects / impact |  |  |
| Environmental impact | - Soil erosion risk - Possible adverse impact on bio-diversity - Loss of plants, shrubs etc - Possible partial loss of soil fertility and may, in some cases, remove the entire fertile layer. The disposal or storage of waste arising from the implementation of this option may have an environmental impact. However, this should be minimised through the control of any disposal route and relevant authorisations. |  |
| Social impact | - Adverse aesthetic effect of removal, even if replaced - Access to public areas may need to be restricted temporarily before turf and topsoil removal is implemented and afterwards while grass grows / turf settles. - Waste disposal may not be acceptable. - Loss of public amenities. |  |
| Practical experience | Tested on semi-large scale (ca. 400 m2) on several occasions in the former Soviet Union. Carried out on a large scale by the Russian authorities after the Chernobyl accident, but not optimised with respect to contaminant distribution, and not carried out consistently over a large area. |  |
| Key references | Andersson et al (2003); Brown and Jones (2000); Brown, Charnock and Morrey (2003); Fogh et al (1999); NRPB (2005); Roed et al (1996) |  |