| ID:55 |  | - specialised surfaces (partucularly metals) in industrial buildings |
| --- | --- | --- |
| Filter removal |  |  |
| Other benefits | Will remove contamination from the area and prevent redistribution of contamination in buildings. |  |
| Countermeasure description | A significant quantity of the radioactivity can be removed by exchanging the filters from industrial buildings, mainly from ventilation systems and other simple fans and heaters. In addition, the removal of the filter from the vehicles (trucks, transport vehicles) can also be effective. |  |
| Target surface or population | Highly contaminated (industrial) ventilation systems. May also be suitable for commerial vehicles. |  |
| Target radionuclides | All radionuclides. Should not be considered for removal of short-lived radionuclides alone. |  |
| Scale of application | Could be carried out on a medium scale in highly contaminated industrial areas. |  |
| Timing of implementation | Maximum benefit if carried out shortly after deposition. Can have a significant effect on reducing contamination levels even if applied a decade after contamination. |  |
| Constraints on implementation |  |  |
| Legal | Liabilities for possible damage to property. |  |
| Environmental / technical |  |  |
| Effectiveness |  |  |
| Reduction in contamination on the surface | Expected reduction up to 100 % |  |
| Reduction in surface dose rates | No estimates made. |  |
| Reduction in resuspension | No estimates made. |  |
| Averted doses | Doses received by members of the public working in these buildings will depend on a number of factors, in particular the amount of time spent in the vicinity of ventilation systems, fans or heaters. No estimates of averted doses have been made. Reductions in dose rates to drivers of vehicles are likely to be higher than those to people working in buildings due to their proximity to the filters. |  |
| Additional doses | Exposure pathways workers could be exposed to are: - External exposure from environment and contaminated equipment - Enhanced resuspension leading to inhalation of dust - Inadvertent ingestion of dust from workers' hands Contributions from pathways in italics are unlikely to be significant and doses from these pathways can be controlled by using PPE. Exposure routes from transport and disposal of waste are not included. The dose over a day to a worker implementing decontamination may be significantly higher than that to an individual living or working in the contaminated area because very high contamination levels can build up in ventilation systems, especially in filters. No illustrative doses have been estimated as they will be very specific to the type of contamination, the tasks undertaken by an individual, controls placed on working and the use of PPE. Dose rates must be assessed prior to any time-consuming activities. |  |
| Factors influencing effectiveness of procedure (technical) | Dependant on the filter contamination (effected during the passage of the air activity), position, type of the filter and filter housing design. |  |
| Factors influencing effectiveness of procedure (social) |  |  |
| Requirements |  |  |
| Required specific equipment | Dependant on the type of the filter system. Ventilation systems and vehicles may require different types of hand tools to be used. |  |
| Required utilities and infrastructure | Transport vehicles for equipment. Scaffolding or mobile lifts for tall buildings. Power supply. |  |
| Required consumables |  |  |
| Required skills | Skilled personnel required. |  |
| Required safety precautions | In tall buildings, where ventilation systems are mounted high up, lifelines are required. Safety helmets. Water proof safety clothing. Respiratory protection is recommended. |  |
| Waste |  |  |
| Amount and type | Filter (solid). |  |
| Intervention costs |  |  |
| Equipment | Hand tools: probably ca. 100 – 500 EURO. Variable costs for scaffolding/lifts according to need. |  |
| Consumables | Power and fuel for equipment transport; at current prices. New filter: depending on the type of filter. |  |
| Operator time | Between a few minutes and a few hours per filter, depending on the type. |  |
| Factors influencing costs | The following factors influence the time taken to implement the option and hence labour costs: - Need for scaffolding /mobile lifts. - Different types of filter and access depending on ventilation system Cost of specialist labour |  |
| Side effects / impact |  |  |
| Environmental impact | The disposal or storage of waste arising from the implementation of this option may have an environmental impact. However, this should be minimised through the control of any disposal route and relevant authorisations. Electronic parts may be damaged by water if not dismounted, Loss of lifetime of equipment due to the mechanical impact (e.g. the basic material will be thinner and rough). |  |
| Social impact | - Acceptability of disposal of contaminated waste. Removal of the corrosion products from the surface; the ventilation system is cleaned, and is expected to run better Reassurance of employees and users and maintaining continuity of work. |  |
| Practical experience | Tested in a number of industrial buildings in the CIS and Europe, after the Chernobyl accident. |  |
| Key references | BMU (2000); Eged et al (2003) |  |